Shogun Facts, Tips & Tricks

Compiled by Kevin Smith

Note: Parts of this information are likely to change on a regular basis

Available from Area 44 of Apricot Insight On-line

This manual assumes that you have completed all Shogun training courses.

Table of Contents

1. BIOS and Software

1.1. Released

1.2. Beta

1.3. BIOS Upgrades

1.3.1. Upgrading Flash Disk

1.3.2. Upgrade Motherboard , User & SMIC BIOS

1.3.3. Upgrading SMC / FPSC

2. Parts containing Flash BIOS

3. Default Configuration

3.1. Clearing CMOS

3.2. F1 Set-up

3.3. 7870 Set-up

3.4. 2940 Set-up

3.5. DPT Set-up

3.6. Running SCU

3.7. Configuring System Board (SCU Rel1 3.11)

3.8. Configuring System Board (SCU Rel 2 3.20)

3.9. Configuring 3c579 (SCU Rel1 3.11)

3.10. Configuring SMIC

3.11. Configuring On-board SCSI controller A

3.12. Configuring On-board SCSI controller B

3.13. Configuring 2940w in Slot 8 (PCI 4)

3.14. Configuring 2940w in Slot 7 (PCI 3)

3.15. Configuring 2940w in Slot 2 (PCI 2)

3.16. Configuring DPT controller in Slot 8 (PCI 4)

3.17. Configuring DPT controller in Slot 7 (PCI 3)

3.18. Configuring 3c595 in Slot 1 / 2 (PCI 1 or PCI 2)

3.19. Locking configurations in SCU

3.20. Saving configuration in SCU

3.21. Resetting the SMC / FPSC

4. Management Software

4.1. SMA

4.2. Environment Pack

4.2.1. Novell NetWare

4.2.2. Installing the AMS

4.2.3. NetWare Loadable Modules

4.2.4. Other files

4.2.5. Command line parameters

4.2.5.1. SMC Last Client

4.2.5.2. SMC Raid Client

4.2.6. KeyLOC Card Registration Utility

4.3. Microsoft Windows NT

4.3.1. Installing the AMS

4.3.2. Removing AMS packages

4.3.3. Windows NT packages

4.3.4. Other files

4.3.5. KeyLOC Card Registration Utility

4.4. SCO OpenServer

4.4.1. Installing the AMS

4.4.2. Removing AMS packages

4.4.3. OpenServer packages

4.4.4. Other files

4.4.5. Command line parameters

4.4.5.1. Network Agent

4.4.5.2. Event Manger

4.4.6. KeyLOC Card Registration Utility

5. General

5.1. PCI Allocation

5.2. IRQ Allocation

5.3. Errors / Warnings

5.3.1. Errors reported with Multiple 3c595 cards

5.3.2. 0804:PCI ROM not found , may be OK for this card: BUS 00,Device0D Function 00

5.3.3. 3Com 3C59X errors in a Shogun running NetWare

5.3.4. SMI Test failed

5.3.5. Front Panel Communication Failure

5.3.6. Processor Speed incorrect

5.3.7. Possible Fault with Shogun System Management Controller

5.4. Mother Board Identification

5.5. Shogun Power distribution board upgrade (Quad processor support)

5.6. Drive Bays

5.7. Additional SCSI Cable Configurations

5.7.1. DPT SCSI Cable Configuration

5.7.2. Adaptec SCSI Cable Configuration

5.8. Shogun Boot Sequence

5.8.1. Default Boot Sequence

5.8.2. Additional 2 x 2940

5.8.3. Additional 3 x 2940

5.8.4. Single DPT Controller

5.8.5. Dual DPT SCSI Controllers

5.9. FANS

5.10. Cards

5.10.1. SMIC Card

5.10.2. SMC / FPSC

5.10.2.1. !! BEWARE !!

5.11. UPS

5.11.1. Power Supply Schematic

5.11.2. Weight

5.12. Buttons / Functions

5.13. Manuals

5.14. Win Beep Pro

5.15. SMA Cable

5.16. Shogun Packing Dimensions

5.17. World Port Modem

5.17.1. LED Indicators

6. Tape Support

6.1. Tape Support Matrix

6.2. Arcserve 5.01g poor backup speeds with DLT

6.3. Arcserve 5.01g and Greater than 16MB ram

6.4. Arcada Backup EXEC for NT v6.0

6.5. Using DLT 4000 SCSI Tape Drive on SCO OpenServer 5

6.6. DLT Jumper Settings

6.6.1. SCSI ID

7. Windows NT

7.1. Adaptec BIOS Settings

7.2. Intial installation

7.3. Hanging during install with a DPT Controller

7.4. Windows NT 3.51 hanging during installation on Adaptec based Shogun

7.5. Error in Event Log

8. NetWare

8.1. Adaptec BIOS Settings

8.2. AIC7870 load order after installation

8.3. Multiprocessor Shogun with current releases of NetWare

8.4. NetWare Not Auto Registering Shogun Memory Correctly

9. OpenServer 5

9.1. 3Com 3C59x Series Ethernet Cards and SCO OpenServer 5 / UNIX 3.2v4.2

9.2. Problem installing SCO OpenServer 5 on Shogun

9.3. Shogun SCO Pack on OpenServer 5

10. DPT RAID configuration

10.1. Recommendations

10.2. Single Array Group across 2 Controllers

10.3. !!BEWARE!!

10.4. Mouse Support with DPT Manager

11. Tricks

11.1. Flash Disk

11.2. SCU

11.2.1. Locking Configurations

11.2.2. Increasing speed of Boot

# BIOS and Software

## Released

| Description | Disk | Version | Part No |
| --- | --- | --- | --- |
| Mother Board BIOS |  | 1.00.02BG0S |  |
| User Profile BIOS |  | 1.00.00 |  |
| SMIC BIOS |  | 1.00.00 |  |
| FPSC BIOS | Rel 2 | 3.008 | 155222SW |
| DPT BIOS / SmartRom | Rel 2 | BIOS 3.c9 SmartRom 07cH | 155858SW |
| Flash Disk | Rel 2 | 1.01 | 155079SW |
| Diagnostic |  |  |  |
| Shogun F/W Upgrade Disk Contains: Motherboard BIOS 1.00.02bgos User Profile 1.00.00 SMIC BIOS 1.00.00 FPSC BIOS 3.008 | Rel 2 |  | 156071SW |
| SCU | Rel 2 | 3.20 | 155160SW |
| SMA Workstation (2 Disks) | Rel 1.1 | 1.01.03 | 155078SW |
| SCO Environment ( 2 Disks) | Rel 1 |  | 155077SW |
| NetWare Environment 4.1x | Rel 1 |  | 155075SW |
| NetWare Environment 3.x | Rel 1 |  | 156859SW |
| Windows NT Environment | Rel 1 |  | 155074SW |
| SCO Environment | Rel 1 |  | 155077SW |
| DOS Accessory’s | Rel 1 |  | 155073SW |
| TCPIP |  |  |  |
| EZSCSI (Intel) | Rel 1 | 3.11(s3) | 156066SW |
| 7800 Family Manager (Intel) | Rel 1 | 1.10S3 | 156067SW |
| 7800 FMS UNIX (Intel) | Rel 1 | 1.10S3 | 156068SW |
| 7800 FMS UnixWare (Intel) | Rel 1 | 1.10S3 | 156137SW |
| EZSCSI (Adaptec) | Rel 2 | 3.12L | 154933SW |
| 7800 FMS UNIX / UnixWare (Adaptec) | Rel 3 | 1.2 | 154979SW |
| 7800 Family Manager (Adaptec) | Rel 2 | 1.2 | 154979SW |
| DPT Manager S/W | Rel 3 | 1eb | 155164SW |

Apricot  will deliver Intel's version of the 7800 FMS / EZSCSI if you get a base system.

If you have 2940's added , you will then get Adaptec's versions. This is for legal reasons.

Unfortunately Adaptec's versions are now higher than Intel’s  and we would recommend the Adaptec versions.

Intel's work OK for just the 2 on board 7870's

## Beta

| Description | Disk | Version | Part No |
| --- | --- | --- | --- |
| Mother Board BIOS |  |  |  |
| User Profile BIOS |  |  |  |
| SMIC BIOS |  |  |  |
| FPSC BIOS | Beta 3 | 3.011 | 155222SW |
| DPT BIOS / SmartRom |  |  |  |
| Flash Disk |  |  |  |
| Diagnostic | Beta 1a | 301 |  |
| Shogun F/W Upgrade Disk Contains: Motherboard BIOS User Profile SMIC BIOS FPSC BIOS |  |  |  |
| SCU |  |  |  |
| SMA Workstation (2 Disks) | 1.2B |  | 155078SW |
| SCO Environment |  |  | 155077SW |
| NetWare Environment 4.1x | Beta 2b |  | 155075SW |
| NetWare Environment 3.x |  |  |  |
| Windows NT Environment | Beta 2b |  | 155074SW |
| SCO Environment (2 Disks) |  |  |  |
| DOS Accessory’s |  |  |  |
| TCPIP | Beta 1b |  |  |
| EZSCSI (Intel) |  |  |  |
| 7800 Family Manager (Intel) |  |  |  |
| 7800 FMS UNIX (Intel) |  |  |  |
| 7800 FMS UnixWare (Intel) |  |  |  |
| EZSCSI (Adaptec) |  |  |  |
| 7800 FMS UNIX / UnixWare (Adaptec) |  |  |  |
| 7800 Family Manager (Adaptec) |  |  |  |
| DPT Manager S/W |  |  |  |

## BIOS Upgrades

All BIOS can be upgraded locally via floppy with the Shogun Firmware upgrade disk.

Upgrades via the Flash disk (F2 on Boot) have been disabled with REL 1 boot introduced with Rel 2.

FPSC can be programmed via SMA or with Rel 2 of the Flash Disk.

### Upgrading Flash Disk

To upgrade the flash disk boot for the floppy (ensure the disk is not write protected) then follow instructions or select Upgrade Flash disk from the Flash disk menu.

### Upgrade Motherboard , User & SMIC BIOS

Boot from the Shogun Firmware upgrade disk and follow the menus or upgrade from the Flash disk Utility menu.

### Upgrading SMC / FPSC

To upload new SMC firmware

1.	Select the appropriate Server window.

2.	Choose Upload Firmware from the Control menu.

The Upload Firmware dialog box appears.

3.	Select the binary file corresponding to the new SMC firmware (it should have a .FRM file extension) and choose OK. (155175SW.BIN or 3006.FRM).

4.	If you are uploading over the network, check the LoginUser variable (in the Users subsystem) to see if anyone currently has a serial connection to the SMC's modem port. If so, you may have to force that user to disconnect, as described in another topic.

- Change the value of the FPSCResetRequest variable to "Active".

- Wait until FIRMWAREUPGRADEFLAG Flag: =FPSC

7.	Choose Power/Reset from the Control menu.

The Power/Reset dialog appears.

8.	Under Action, select the [Power] Off option. Under Scheduled Action, select the Enabled check box and specify a time a few minutes into the future. Choose OK.

Note: You cannot schedule a Power On to occur after the SMC firmware has been upgraded. The upgrading process does not preserve this information.

9.	Close the Server window to disconnect from the server.

As soon as the server is in Power Off mode, and provided there is no serially-connected workstation, the SMC updates its firmware using the binary file you have given it..

The Shogun will display 0001 on the LCD when upgrade is complete.

To upgrade the FPSC via Flash Disk

Boot from the Flash Disk “F2”

Select Upgrade SMC Firmware

Insert disk with the FPSC binary on.

If the file from the FPSC disk is used you will have to rename 155175SW.BIN to the BIOS version with an extension of FRM

E.g. REN 155175SW.BIN 3008.FRM

Transfer file and Upgrade

Wait until the machine switches off and the LCD reports 0001.

# Parts containing Flash BIOS

The following PCB’s all contain Flash devices and must be reprogrammed if ordered as spare parts.

| Part | Flash Device |
| --- | --- |
| Mother Board | Motherboard BIOS |
|  | User Binary |
| System Management Interface Card (SMIC) | SMIC BIOS |
|  | Flash Disk |
| System Management Controller (SMC or FPSC) | FPSC BIOS |

The necessary Firmware can be found and upgraded in various ways.

| Disk Containing Firmware | Part Number |
| --- | --- |
| Firmware Update Disk | 156071SW |
| Flash Disk Utility | 155079SW |
| FPSC Firmware | 155222SW |

Methods for Upgrading.

| Flash Device Upgraded | Method |
| --- | --- |
| FPSC | SMA |
| Mother Board, User Binary & SMIC | SMA Flash disk in Pass-through Mode |
| Flash Disk | Flash Disk |
| All BIOS | Flash Disk “F2” |
| Mother Board, User Binary & SMIC | Firmware Update Disk |

All Disk’s Containing Firmware can be located in Area44 of Apricot Insight on-line or on the Apricot Insight CD.

# Default Configuration

Listed below are the steps to restore a Shogun to it’s default configuration.

## Clearing CMOS

Check that the switch in DIL-switch S8C1-1 on the motherboard is closed (up).

Power on the system and wait for the message “0905 : NVRAM Cleared by Jumper” or if this message never appears because the system continually reboots after the message “CMOS changed by SMIC BIOS - Rebooting...”

Shutdown the system.

Open the CMOS CLEAR switch S8C1-1 (down).

Power on the system

Wait for the message “CMOS changed by SMIC BIOS - Rebooting...”

Then shutdown the system.

## F1 Set-up

When machine restarts, press <F1> during memory check to enter Set-up.

In Main menu move cursor to System Date and System Time and press <Enter> to update.
Move cursor right to enter Advanced menu .

Press <Enter> for Peripheral Configuration

Change Configuration Mode to “Manual”

Change Standard IDE Interface to “Disabled”.

Press <Esc> to exit.

Move down to Advanced Chipset Configuration and enter sub-menu.

Change Force PCI Concurrency to “Enable”.

Press <F10> and then <Enter> to save and reboot.

## 7870 Set-up

During Adaptec 7870 SCSI sign-on press <Ctrl-A> to enter SCSI-Select.
Select each controller in turn (01:0Dh & 01:0Eh) and select Configure/View Host Adapter Settings.

Press <F6> and select “Yes” to reset to defaults.

Move to SCSI Device Configuration and press <Enter> to reach set-up screen.

Change Initiate Wide Negotiation from “no” to “yes” for devices #0, #1, #3, #4 and #7.

Move to the bottom line and change Include in BIOS Scan  from “yes” to “no” for devices #8 to #15.

Press <Esc> twice and select “yes” to save.

Press <Esc> to return to the BIOS sign on screen.

Configure the other controller the same.
Press <Esc> to exit and reboot when one or both channels have been updated.

## 2940 Set-up

***  IGNORE THIS PARAGRAPH IF NO 2940 CARDS ARE FITTED  ***
During Adaptec 2940 SCSI sign-on press <Ctrl-A> to enter SCSI-Select.

Select each controller in turn ( 00:0Dh, 01:0Ch & 01:0Fh) and select Configure/View Host Adapter Settings.

Press <F6> and select “Yes” to reset to defaults.

Move to SCSI Device Configuration and press <Enter> to reach set-up screen.

Change Initiate Wide Negotiation from “yes” to “no” for devices #2, #5 and #6.

Change Send Start Unit Command from “no” to “yes” for devices #0 to #7.

Move to the bottom line and change Include in BIOS Scan  from “yes” to “no” for devices #8 to #15.

Press <Esc> twice and select “yes” to save.

Press <Esc> to return to the BIOS sign on screen.

Configure the other controller the same.
Press <Esc> to exit and reboot when one or all channels have been updated.

## DPT Set-up

***  IGNORE THIS PARAGRAPH IF NO RAID CARDS ARE FITTED  ***
During DPT RAID sign-on press <Ctrl-D>.

Press <F9> to select default settings.  Press <y> to save and reboot.

## Running SCU

Run Configuration Utility (SCU), followed by Run SCU. SCU can be run from Floppy or Flash Disk.

Press <Enter> to bypass ***WARNING*** message.

Select  Step 3 : Change Configuration Settings.

Press <Esc> to cancel Password Menu when it appears.
Press <Insert> and <Enter> to add “!ACL0100.CFG - System Management Interface Card”.

Select Slot 2, <Enter>.

## Configuring System Board (SCU Rel1 3.11)

Press <Enter> to select System Board.

Move cursor down to Peripheral Configuration Group

Change Auto Configuration to “Disable”.

Move down to On-Board Disk Controllers and change On-Board IDE Controller to “Disable”.

Move down to On-Board Communication Devices.

If 3c579 card is fitted Change Parallel Port Configuration to “Disable“

Change Parallel Port Configuration to “LPT1 : 378h - IRQ 7 “.

Change Serial Port 2 Configuration to “Disabled”.

Change Serial Port 1 Configuration to “COM1 : 3F8h - IRQ4”.

Return to Serial Port 2 Configuration and change it to “COM2 : 2F8h - IRQ3”.

Move cursor down to the Boot Subsystem Group

Change Display”<F1>” for set-up message during post to “Disable”

Change Require User Interaction on POST Errors to “Disable”.

Move down to the Management Subsystems Group

Change System management Mode to “Enable”.

Change Event Logging to “Enable”.

Move up to A/D Channel Enable Switch and press <Enter>, <Enter>.

Press <space> to cancel the first tick.

Move the cursor down and press <space> to cancel the other 5 ticks (No ticks should be displayed).

Press <Enter>  then , <Esc> to return to the Main Menu.

## Configuring System Board (SCU Rel 2 3.20)

Press <Enter> to select System Board.

Move down to On-Board Disk Controllers and change On-Board IDE Controller to “Disable”.

Move cursor down to the Boot Subsystem Group

Change Display”<F1>” for set-up message during post to “Disable”

Change Require User Interaction on POST Errors to “Disable”.

Press <Enter>  then , <Esc> to return to the Main Menu.

## Configuring 3c579 (SCU Rel1 3.11)

If the system has a 3c579 EISA card fitted

Press <Enter> to select 3com 3c579 EISA Network Adapter

Press <Enter> change Interrupt Request level = “5”

Press <Esc> to return to configuration screen

Press <Enter> to select System Board.

Move down to On-Board Communication Devices.

Change Parallel Port Configuration to “LPT1 : 378h - IRQ 7 “.

Press <Esc> to return to configuration screen.

## Configuring SMIC

Press <Enter> to select System Management Interface Card

Move down to Pass Through press <Enter>

Select “IRQ14, COM3 port address” from menu.

Move down to Diagnostic Port and press <Enter>.

Select “IRQ15” from menu.

Press  <Esc> to return to configuration screen.

## Configuring On-board SCSI controller A

Move down to the PCI SCSI Device at Bus 1 Dev D and press <Enter>.

Press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 9.

Press <Enter> , <Esc> to return to configuration screen.

## Configuring On-board SCSI controller B

Move to the PCI SCSI Device at Bus 1 Dev E and press <Enter>.

Press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 9.

Press <Enter> , <Esc> to return to configuration screen.

## Configuring 2940w in Slot 8 (PCI 4)

If the system has one or more 2940W controllers fitted move to the PCI SCSI Device at Bus 1 Dev F and press <Enter>.

Press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 9.

Press <Enter> , <Esc> to return to configuration screen.

## Configuring 2940w in Slot 7 (PCI 3)

If the system has two 2940W controllers fitted move to the PCI SCSI Device at Bus 1 Dev C and press <Enter>.

Press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 9.

Press <Enter> , <Esc> to return to configuration screen.

## Configuring 2940w in Slot 2 (PCI 2)

If the system has two 2940W controllers fitted move to the PCI SCSI Device at Bus 0 Dev D and press <Enter>.

Press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 9.

Press <Enter> , <Esc> to return to configuration screen.

## Configuring DPT controller in Slot 8 (PCI 4)

If the system has one or more RAID controllers fitted move to the PCI SCSI Device at Bus 1 Dev F and press <Enter>.

Press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 10.

Press <Enter> , <Esc> to return to configuration screen.

## Configuring DPT controller in Slot 7 (PCI 3)

If the system has two RAID controllers fitted move to the PCI SCSI Device at Bus 1 Dev C and press <Enter>.

Press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 10.

Press <Enter> , <Esc> to return to configuration screen.

## Configuring 3c595 in Slot 1 / 2 (PCI 1 or PCI 2)

If the system has one or more Ethernet cards fitted move to the PCI SCSI Devices at Bus 0 Dev C or Bus 0 Dev D.

For each device press <Enter> and change to “Manual Configuration”.

Press <F6> to examine resources.

Press  <+> or <-> on numeric keypad until IRQ is set to 11.

If two cards are fitted ensure they are set on different IRQ’s.

IE 11 and 5

Press <Enter> , <Esc> to return to configuration screen.

## Locking configurations in SCU

Lock the configurations for each section by pressing “f8” a asterix should appear by each section.

## Saving configuration in SCU

Press <Esc> to return to the Main Menu

Move to “Step 4 : Save Configuration” and press <Enter>

When saving has completed select “Step 6 : Exit” and press <F10> to reboot.

## Resetting the SMC / FPSC

When the machine has rebooted reset the System Management controller.

Hold down CONTROL + STANDBY + RESET on the front panel until the LCD displays 8.8.8.8

Hold down STANDBY + RESET (Until Display Clears & Beep stops)

# Management Software

## SMA

SMA works at a maximum of 19200 BPS

If you are unable to connect you might have to set the COM port to 19200 via control panel in Windows.

For reliable serial connection under WFWG 3.11 patch WG1001.EXE in Area6 will need to be installed.

First connection can be made Directly or via LAN

Default Users

Network

NETUSER,PASSWORD

Serial Direct

PORTUSER,PASSWORD

SMA versions > Rel 1 will have case sensitive user names and passwords

When closing a connection if SMA reports communications in progress don't quit because SMA and the shogun are still communicating and any changes can be lost.

If the user is set to Force Traps Shogun doesn’t close the connection to SMA after establishing the connection. This is with 3.006 FPSC and Rel 1 SMA.

The event manager script does not sync with setting made with SMA.

SMA will lock the COM ports that have been configured until you quit out of SMA. I.E. If you have a modem on COM 2 and SMA has a configuration for COM 2 no other application will be able to use this (fax , Winbeep). This is because if a Shogun forces a trap it need to open a connection to SMA.

## Environment Pack

### Novell NetWare

The Apricot Management Suite for NetWare is supplied as a set of NetWare Loadable Modules (NLMs) on the Novell NetWare Support Disk. The disk supports NetWare v4.0 (or later).

### Installing the AMS

To install the AMS:

- 1.	At a workstation, insert the Novell NetWare Support Disk in drive A.

- 2.	Connect to the server and login as a user with Supervisor rights.

- 3.	Map a drive to the SYS:SYSTEM directory.

- 4.	In DOS, change to drive A and type

- NWSETUP drive:

- replacing drive with the letter of the drive that you mapped at Step 3.

The AMS software is installed automatically and the server’s AUTOEXEC.NCF file is amended to load it.

To prevent the software autoloading every time the server boots, remove the APRICOT command from the AUTOEXEC.NCF file.

### NetWare Loadable Modules

| Generic name | NetWare name | NLM |
| --- | --- | --- |
| SMC Device Driver | Apricot FPSC Device Driver for NetWare | FPSCDD.NLM |
| Network Agent |  | N/A |
| SMC LAN Client | Apricot Last Client for NetWare | NWLAND.NLM |
| SMC Raid Client | Apricot RAID Daemon | RDAEMON.NLM |
| SMC ECC Client |  | N/A |
| SMC Last Client | Apricot Last Client for NetWare | NWLASTC.NLM |
| EM Trap Handler |  | EMANAGER.NLM |
| EM UPS Service | Event Manager UPS Module for NetWare | EMUPS.NLM |
| EM Standby/Reset Service | Event Manager Shutdown/Reset Module for NetWare | EMRES.NLM |
| EM Environment Service | Event Manager Environment Module for NetWare | EMENV.NLM |
| EM Security Service | Event Manager Security Module for NetWare | EMSEC.NLM |
| EM Security Client | Apricot Security Client for NetWare | NWSECURE.NLM |
| SNMP Extension Agent | Apricot SNMP Extension Agent for NetWare Apricot SNMP Extension Agent Master Controller for NetWare | APRSNMPA.NLM APRSNMPC.NLM |
| KeyLOC Card Registration Utility | Apricot IR Registration Application | IRCARD.NLM |

### Other files

| Generic name | NetWare name | Filename |
| --- | --- | --- |
| EM Script File | Apricot Event Manager Script File for NetWare | EMANAGER.PRG |
| EM Event Mapping File | Apricot Event Manager Trap Mapping File for NetWare | EMANAGER.DAT |
| EM Log File (default) | N/A | EMANAGER.LOG |

### Command line parameters

#### SMC Last Client

The SMC Last Client NLM has command line switches that allow you to alter the value of the watchdog timer (WDT) and the frequency at which it is updated or refreshed:

LOAD NWLASTC VALUE=wdtsecs DELAY=updatesecs

where wdtsecs and updatesecs are expressed in seconds. The watchdog timer must be at least 10 seconds longer than the refresh time. The defaults are a watchdog timer of 300 seconds and an update time of 120 seconds.

#### SMC Raid Client

The SMC Raid Client has a command line parameter that specifies the interval at which the client polls the Raid controllers:

LOAD RDAEMON /T:pollsecs

where pollsecs is the polling interval in seconds. The default interval is 300 seconds (5 minutes).

### KeyLOC Card Registration Utility

To register a KeyLOC card at the server:

- 1.	At the console prompt, type

- IRCARD index_num

- where index_num is the index number (1 to 16) of the KeyLOC card you want to register.

- 2.	Aim and fire the KeyLOC card at the infrared sensor on the front of the server.

KeyLOC cards can also be registered by using the SMA. See the SMA User’s Guide for more information.

## Microsoft Windows NT

The Apricot Management Suite for Windows NT is supplied as a set of packages on the Windows NT Support Disk. The disk supports Windows NT v3.51 (or later).

### Installing the AMS

To install the AMS:

- 1.	Connect a workstation to the server and log on as a user belonging to the Administrator group or logon at the server as a administrator.

- 2.	If you are installing the software from somewhere other than drive A, at the command prompt type:

- ntconfig -s [drive:]path

- 3.	To install the complete AMS, type :

- ntconfig -i

- To install individual packages, you can use the format

- ntconfig -i packages

- where packages is a list of one or more of the packages shown in the table overleaf.

- The installation process should proceed with a final message indicating successful installation.

- 4.	Reboot the server for the change to take effect.

### Removing AMS packages

To remove AMS packages:

- 1.	At the command prompt, type

- ntconfig -r packages

- replacing packages with a list of the packages you want to remove.

- 2.	Reboot the server for the change to take effect.

### Windows NT packages

| Generic name | Windows NT name | Package | Filename |
| --- | --- | --- | --- |
| SMC Device Driver | Apricot Windows NT FPSC Device Driver | fpsc | FPSC.SYS |
| Network Agent | SMA Agent | smaagent | NETA.EXE |
| SMC LAN Client | LAN Monitor | lanmonitor | LANM.EXE |
| SMC Raid Client | RAID Monitor | raidmonitor | RAIDM.EXE |
| SMC ECC Client | ECC Monitor and ECC Device Driver | ecc | ECCM.EXE ECCDD.SYS |
| SMC Last Client | Last Client | lastclient | LASTC.EXE |
| Event Manager | Apricot Windows NT Event Manager | eventmanager | EMANAGER.EXE |
| EM Security Client and KeyLOC Card Registration Utility | Security Monitor, Security KeyWait, Apricot Security Driver for Windows NT | aprsecm aprsec | SECM.EXE SECK.EXE APRSEC.SYS REGISTER.EXE |
| SNMP Extension Agent | SNMP Agent Extension | snmp | APRSNMP.DLL |

### Other files

| Generic name | Windows NT name | Filename |
| --- | --- | --- |
| EM Script File | Apricot Event Manager Script File | EMANAGER.PRG |
| EM Event Mapping File | N/A | EMANAGER.DAT |
| EM Log File (default) | N/A | EM.LOG |

### KeyLOC Card Registration Utility

To register a KeyLOC card at the server:

- 1.	At the console prompt, type

- register index_num

- where index_num is the index number (1 to 16) of the KeyLOC card you want to register.

- 2.	Aim and fire the KeyLOC card at the infrared sensor on the front of the server.

KeyLOC cards can also be registered by using the SMA. See the SMA User’s Guide for more information.

## SCO OpenServer

The Apricot Management Suite for OpenServer is supplied as a set of packages on the SCO OpenServer Support Disk. The disk supports OpenServer 5.0 (or later).

### Installing the AMS

To install the AMS:

- 1.	From the desktop, start the Software Manager program.

- 2.	Select Install New from the Software menu.

- 3.	Select the current host followed by the media device where the software is located and then insert the disk.

- 4.	To install the entire contents of the diskette, choose the Full button. To install individual packages, choose the Partial button, then select the required software from the list and choose the Install button (see the table overleaf for a list of the packages).

You can also install packages using the text-mode custom utility.

### Removing AMS packages

To remove AMS packages:

- 1.	Start Software Manager.

- 2.	From the list of installed software select the software that you wish to remove.

- 3.	Choose Remove Software from the Software menu.

### OpenServer packages

| Generic name | OpenServer name | Package |
| --- | --- | --- |
| AMS for UnixWare | SCO OpenServer Pack | sup |
| SMC Device Driver | FPSC Device Driver | fpsc |
| Network Agent | LAN Client | N/A |
| SMC LAN Client | LAN Statistics Client | land |
| SMC Raid Client | Raid Disk Client | raid |
| SMC ECC Client | ECC Client and Device Driver | ecc |
| SMC Last Client | Last Client | lastc |
| Event Manager | Event Manager | eman |
| EM Security Client and KeyLOC Card Registration Utility | Security Screen Blanker Device Driver and Client | sbd |
| SNMP Extension Agent | SNMP Extension | smux |

### Other files

| Generic name | OpenServer name | Filename |
| --- | --- | --- |
| EM Script File | N/A | emanager.prg |
| EM Event Mapping File | N/A | emanager.dat |
| EM Log File (default) | N/A | em.log |

### Command line parameters

#### Network Agent

The Network Agent can be stopped and re-started from the command line:

netagent stop

netagent start

#### Event Manger

The Event Manager can be stopped and re-started from the command line:

emanager stop

emanager start

### KeyLOC Card Registration Utility

To register a KeyLOC card at the server:

- 1.	At the console prompt, type

- irreg index_num

- where index_num is the index number (1 to 16) of the KeyLOC card you want to register.

- 2.	Aim and fire the KeyLOC card at the infrared sensor on the front of the server.

KeyLOC cards can also be registered by using the SMA. See the SMA User’s Guide for more information.

# General

## PCI Allocation

PCI BUS 0 Top slots

PCI BUS 1 Bottom slots

PCI device		Physical slot

Bus 0 Device C	PCI Card 1	Slot 1

Bus 0 Device D	PCI Card 2	Slot 2

Bus 1 Device C	PCI Card 3	Slot 7

Bus 1 Device D	SCSI Controller A	On-board

Bus 1 Device E	SCSI Controller B	On-board

Bus 1 Device F	PCI Card 4	Slot 8

## IRQ Allocation

IRQ	0	timer

1	keyboard

2	cascade

3	com2

4	com1

5	3C579

6	floppy

7	parallel port

8	RTC

9	All Adaptec 7870's (including on AHA2940 cards).

10	All DPT cards

11	3C595

12	Mouse

13	Co pro

14	SMIC COM3	(note onboard IDE needs disabling)

15	SMIC Diagnostic Port

## Errors / Warnings

### Errors reported with Multiple 3c595 cards

If you use more than 1 3c595 Ethernet card the cards must have unique interrupts or possible NVRAM corruption can occur. We recommend IRQ 11 and IRQ 5.

### 0804:PCI ROM not found , may be OK for this card: BUS 00,Device0D Function 00

If  a 3c595 card is installed in a Shogun the above warning will occur. This is purely a warning as the machine is trying to initialise the option ROM which is not installed.

### 3Com 3C59X errors in a Shogun running NetWare

3Com 3C595 and 3C590 now supported in a Shogun running NetWare.

The initial problem stated in IPB 2238 were the 3c595 and the 3c590 in some

circumstances may cause problems under NetWare has now been fixed.

To rectify the problem Apricot and 3Com have released a new version of 3c59x.LAN.

The LAN and associated files can be located in Area 44 of Apricot Insight On-Line (3c59xSH.EXE)

File versions:

3C59X.LAN

3Com Fast EtherLink 3C59X / EtherLink III 3C59X

Version 4.02d  Feb. 2, 1996

(C) Copyright 1995 3COM Corporation. All Rights Reserved.

ETHERTSM.NLM

Novell Ethernet Topology Specific Module

Version 2.50   Nov. 29, 1994

Copyright 1994 Novell, Inc.  All rights reserved.

MSM.NLM

Novell Generic Media Support Module

Version 2.50   Nov. 23, 1994

Copyright 1994 Novell, Inc.  All rights reserved.

Later releases of this file will be included on future releases of the 3c59x EtherDisk (Later than Rel 5.1)

### SMI Test failed

On boot if the error

***SMI TEST FAILED

***CRITICAL ERROR LOGGING TEST FAILED

Run SCU (Rel1 3.11) and under SYSTEM BOARD

Move down to the Management Subsystems Group

Change System management Mode to “Enable”.

Change Event Logging to “Enable”.

Move up to A/D Channel Enable Switch and press <Enter>, <Enter>.

Press <space> to cancel the first tick.

Move the cursor down and press <space> to cancel the other 5 ticks (No ticks should be displayed).

### Front Panel Communication Failure

On boot if the error

*** Front Panel Communication Failure

Run SCU (Rel 3.11)

Press <Enter> to select System Management Interface Card

Move down to Pass Through press <Enter>

Select “IRQ14, COM3 port address” from menu.

Move down to Diagnostic Port and press <Enter>.

Select “IRQ15” from menu.

Press  <Esc> to return to configuration screen.

### Processor Speed incorrect

SMA and SCU  will report that the processor speed is 100 MHz not 133 MHz on Cyclops systems.

This has been fixed with Release 2 of BIOS (1.00.02BG0S)

### Possible Fault with Shogun System Management Controller

Fault

During the production of the System Management Controller Card a small quantity were fitted with an EPROM instead of a Flash ROM device. Thus when you try to upgrade the BIOS on the System Management controller it will fail, because you cannot reprogram an EPROM using a Flash Utility.

Machine affected

The faulty System Management Controller was possibly fitted into machines before a factory purge on the 29th November 1995. Apricot are suspecting all Shoguns prior to the purge on the 29th November.

Identification

The cards can be identified by physically checking the device or by trying to reprogram the FPSC BIOS.

Upgrade Identification

After reprogramming the FPSC BIOS SMA will report  FIRMWAREUPGRADEFLAG = FAILED or by checking the FPSC BIOS via the Front Panel, it will remain at 3.006.

Physical identification

The device (IC6) should have been a Flash device (28F010) and not a EPROM device which fitted in error (27C010).  The IC is located behind Fan 4.

Action to take

TSD have a list of machines and serial numbers of machines that were shipped prior to the factory purge and might have faulty System Management Controllers fitted.

If the machine has a faulty board nothing major will happen as all software will work with 3.006 FPSC BIOS enabling the board to be replaced at a later stage thus enabling the new functionality of later BIOS releases.

If the board is faulty it will be exchanged by Apricot.

## Mother Board Identification

Mother boards can de identified by the PBA number situated behind the slots on the mother board. The first 6 digits defines the board the 3 digits after the slash refer to build level.

FAB5A 	637919-005

FAB5	634688-003

## Shogun Power distribution board upgrade (Quad processor support)

Check before fitting a second dual CPU card

In Shogun systems fitted with the Pentium motherboard it is important to check, prior to fitting a second dual CPU card, that the Power Distribution Board is REV D or later. Early Shogun models were supplied fitted with a REV B Power Distribution Board.

The REV B board is satisfactory in systems with two single CPU cards, or one dual CPU card. However, the fitting of a second dual CPU card requires that the system be upgraded to the REV D or later Power Distribution Board.

For information about the location and possible removal of this board please refer to the detailed instructions in the Owner’s Handbook.

Revision differences

The revision markings of the Power Distribution Board are not easily viewable when  the board is in situ, but there is one important feature which makes it readily identifiable:

- The REV B Power Distribution Board has a low profile encapsulated voltage regulator module in the centre of the board. Module size approx. 50 mm square and 12 mm high.

- The REV D Power Distribution Board has a large multi-finned heatsink approx. 80 x 100 mm and 35 mm high, the edge of which is at the side of the board, and it is immediately in view when the inner cover plate is removed.

As confirmation, on removal, the board silk screening will have the version marked as /B or /D on the component side, at the end nearest the computer front.

Apricot TSD also have a list of serial numbers of those machines initially installed with REV B Power Distribution Boards.

The replacement Part Number for REV D Power Distribution Board is: 12365331

## Drive Bays

J1 Fitted for (Delay start) Default for all bays

J2 Fitted for bay B this disables termination from standard (wide) SCSI thus a termination resistor pack is fitted to the removable drive media cable.

J2 Removed for all other bays enables termination on drive bay (wide SCSI)

## Additional SCSI Cable Configurations

### DPT SCSI Cable Configuration

| Slot 8 | Connector 1 | Bay B |
| --- | --- | --- |
| Primary Adapter | Connector 2 | Bay A |
|  | Connector 3 | Bay C |
| Slot 7 | Connector 1 | Bay D |
| Secondary Adapter | Connector 2 | Bay E |

### Adaptec SCSI Cable Configuration

| Onboard SCSI A | Bay B |  |
| --- | --- | --- |
| Onboard SCSI B | Bay A |  |
| Slot 8 | Bay C |  |
| Slot 7 | Bay D |  |
| Slot 2 | Bay E | Not a configured option |

## Shogun Boot Sequence

### Default Boot Sequence

The Shogun will boot of Onboard SCSI A then Onboard SCSI B

Note: Removable Media is on Onboard SCSI A.

### Additional 2 x 2940

The Shogun will boot of the 2940 in slot 7 ,Onboard SCSI A, Onboard SCSI B then 2940 in slot 8.

Note: Removable Media is on Onboard SCSI A.

### Additional 3 x 2940

The Shogun will boot of the 2940 in slot 2, slot 7 ,Onboard SCSI A, Onboard SCSI B then 2940 in slot 8.

Note: Removable Media is on the Onboard SCSI A.

### Single DPT Controller

The Shogun will boot from Channel 1, Channel 2 then Channel 3.

Note: Removable Media is on the Channel 1.

### Dual DPT SCSI Controllers

The Shogun will boot from Channel 1, Channel 2, Channel 3 then the Second Card from Channel 1 then Channel 2.

Note: Removable Media is on the Channel 1 on the DPT card in Slot 8.

## FANS

| 1 | Front Bay | Upper |
| --- | --- | --- |
| 2 | Motherboard | Middle |
| 3 |  | Lower |
| 4 | Back Bay | Upper |
| 5 | Drive | Middle |
| 6 |  | Lower |

## Cards

### SMIC Card

Pass Through IRQ14 Com3

FPSC (diagnostic) IRQ 15

Shared memory address d000-d4000 (Default)

Switches all 4 switches ON (Default)

Switch 1 OFF displays debug information

### SMC / FPSC

PL13 Jumper Setting

Fitted 	(Default) SMC / FPSC obtains power from UPS

Removed	Isolates SMC from UPS

#### !! BEWARE !!

The connections from the SMC to the Front Panel and UPS are identical.

If not connected to the correct location this can cause damage to the SMC.

This damage is normally to the buffer chip located at IC48 because UPS battery voltage of 56v is passed to the SMC

UPS

### Power Supply Schematic

### Weight

UPS gross weight 40KG

## Buttons / Functions

| Button | Function |
| --- | --- |
| POWER | Power on the system |
| CONTROL | Silences alarm sound and clears the LCD |
| STANDBY | Issues a STANDBY request to software and starts the hold_off_timer. When the hold_off_timer expires DC power is removed from the motherboard |
| CONTROL + STANDBY | Issues a STANDBY cancel request to software, if the software sends an acknowledgement back the hold_off_timer is cancelled |
| RESET | Issues a reset request to software and starts the hold_off_timer. When the hold_off_timer expires the motherboard is reset. |
| CONTROL + RESET | Issues a reset cancel request to software, if the software sends an acknowledgement back the hold_off_timer is cancelled |
| Multi-button Function Mode |  |
| CONTROL + STANDBY + RESET | System displays 8.8.8.8 on the LCD, this indicates the system has entered Multi-button Function Mode for 10 seconds only |
| CONTROL | Sends a sequence of AT reset commands to the modem |
| STANDBY | The system creates an NMI error causing the NOS to Panic/Abend. This should allow the NOS to do a complete memory dump. |
| STANDBY + RESET | Resets the SMC to defaults. Removes all SMA user information going back to the default NETUSER and PORTUSER |
| CONTROL + STANDBY + RESET | The SMC displays it's BIOS version on the LCD |

## Manuals

| DOC-GETTING STARTED | 15456231 |
| --- | --- |
| HANDBOOK-OWNERS | 15456331 |
| DOC-DIAGNOSTIC CODES | 15456431 |
| DOC-SMA OVERVIEW | 15456531 |
| DOC-APR MANAGEMENT S/W INSTAL | 15456631 |
| GUIDE-USER EVENT MANAGER | 15456731 |
| GUIDE-USER SMA | 15456931 |
| GUIDE-USER SNMP | 15457031 |

## Win Beep Pro

Contact Hutchison Telecom Ltd.

2420 The Crescent

Birmingham Business Park

Solihull

B37 7YE

0121 606 6850

RRP £99

## SMA Cable

| SHOGUN | CLIENT | CLIENT |
| --- | --- | --- |
| (25-PIN Female) | (9-PIN Female) | (25-PIN Female) |
| 2 | 2 | 3 |
| 3 | 3 | 2 |
| 4 | 8 | 5 |
| 5 | 7 | 4 |
| 7 | 5 | 7 |
| 8 | 4 | 20 |
| 20 | 1 | 8 |

## Shogun Packing Dimensions

51 x 92 x 108 cm (width x length x height)

Max Weight 160kg (25 stone)

Remember to leave 2m infront of the packing for the ramp.

## World Port Modem

### LED Indicators

| HS | Connection Speed |  |
| --- | --- | --- |
| Green | ON | 16.8K, 14.4K ,12K bps |
|  | FLASH | 9600, 7200, 4800 bps |
| Red | ON | 2400bps |
|  | FAST FLASH | 1200 bps |
|  | SLOW FLASH | 300 bps |
| ARQ / FAX | Error Control and Fax |  |
|  | ON | Error Control |
|  | FLASH | Fax Mode |
| CD | Carrier Detect |  |
|  | ON | Carrier Detect |
|  | FAST FLASH | Cellular HST (Dual Standard) |
|  | SLOW FLASH | Idle |
| LB | Low Battery |  |
|  | ON | Low Battery |

# Tape Support

## Tape Support Matrix

| Tape Device | UnixWare | SCO | NT | NetWare |
| --- | --- | --- | --- | --- |
| DDS II | 1 | 1 | 1 (2) | 1 (3) |
| DLT 4000 | 1 | 1 | 2 | 3 |
| DDS II autoloader | not supported | not supported | 2 , 4 | 3, 5 |

- Supported in Operating system

- Supported by Arcada Backup Exec ver 6 for NT

- Supported by Arcada Backup Exec ver 7 for NetWare

- Supported by Autoloader Arcada Backup Exec supplement for NT

- Supported by Autoloader Arcada Backup Exec supplement for NetWare

Autoloaders will work as a “large tape drive” if supplement is not purchased

## Arcserve 5.01g poor backup speeds with DLT

As default Arcserve is configured to use a 512k block size. Due to such a small block size being used as default the DLT is unable to stream correctly.

To overcome this problem the block size and buffer size need to be increased.

Edit SYS:\ARCSERVE\NLM\TAPESVR.CFG and add the following lines (changes in Bold).

“SYS:\ARCSERVE\NLM\TAPESVR.CFG”

[CONFIG]

BLOCKSIZE=5

BUFFER=20

[NLM]

NLM1 = standard group-ARCSERVE        ; 1st load

After restarting Arcserve check the block size of the cartridges by the following..

TAPESVR screen - DEVICE MANAGEMENT - TAPE DRIVE STATUS

If the Block size is not 16384 then perform an erase on the tape, this will change the block size from 512 to 16384.

## Arcserve 5.01g and Greater than 16MB ram

To enable Arcserve to use memory above 16Mb the following syntax needs to be amended to SYS:\SYSTEM\ASTART.NCF (changes in Bold)

SYS:\SYSTEM\ASTART.NCF

REM =======================================================

REM   Make sure the following drivers are loaded before you run ASTART

REM   aha1540.dsk or aha1640.dsk or aha1740.dsk , aspitran.dsk

REM =======================================================

Search Add SYS:\ARCSERVE\NLM

LOAD SYS:\SYSTEM\STREAMS

LOAD SYS:\SYSTEM\CLIB

LOAD SYS:\SYSTEM\TLI

LOAD SYS:\SYSTEM\SPXS

LOAD SYS:\SYSTEM\AFTER311

LOAD SYS:\SYSTEM\NWSNUT

LOAD BTRIEVE -p=4096 -f=20 -h=60 -l=20 -t=15 -u=0 -s=30 -m=512

LOAD BSPXCOM -d=8192 -s=15 -w=3

LOAD SYS:\ARCSERVE\NLM\ASDB

LOAD SYS:\ARCSERVE\NLM\FAXLIB

LOAD SYS:\ARCSERVE\NLM\ARCSERVE

LOAD SYS:\ARCSERVE\NLM\ASPI50 host=0 USEABOVE16

LOAD SYS:\ARCSERVE\NLM\TAPESVR USEABOVE16

LOAD SYS:\ARCSERVE\NLM\CLIENTS

## Arcada Backup EXEC for NT v6.0

The evaluation copy of NT backup exec v6.0 LOT NUMBER #DK95108 contains a DLT driver dated 7/20/95 , size 16480 bytes.

A released copy of backup exec for NT v 6.0 LOT NUMBER #DK9590 contains a DLT driver dated 5/2/95 , size 16096 bytes.

The released copy unfortunately does not work with the DLT 4000 drive on either Adaptec or DPT controllers. The driver reports that it cannot start in the event log.

The evaluation copies driver works OK.

However the updated DLTTAPE.SYS has been copied to AREA 44 of the BBS (DLTNT.EXE).

The DLT driver (DEC / Cipher DLT SCSI drivers , All) is used when adding a tape device to NT

Also a maintenance pack (BNT6i108.EXE with docs or BNT6L108.EXE no docs) for Backup EXEC to update to build 108 has been copied to AREA 88 of the BBS

## Using DLT 4000 SCSI Tape Drive on SCO OpenServer 5

When doing cpio backups using the DLT 4000 SCSI tape drive under SCO OpenServer 5, it is important to specify a particular block size using the "-C" option. Whilst the device will still back up if this is not done, it will not stream in an efficient manner.

The effect of this is that it's performance will be greatly reduced, with the result that backups can take up to 7 times longer than is actually necessary.

We therefore recommend that  when cpio is invoked, the blocking factor is set to 10240 bytes to the record. We have tried greater values with no significant improvement in performance. A typical cpio instruction would be as follows:

# find . -depth -print|cpio -ovC10240 >/dev/rct0

In practice, this has resulted in a data transfer rate approaching 50Mb/min, as against some 7Mb/min when cpio uses its default blocking factor.

## DLT Jumper Settings

SCSI ID

# Windows NT

## Adaptec BIOS Settings

Enable > 1GB support

## Intial installation

If a drive is in A0 when the BIOS load’s the machine will boot off the second controller. After the initial install when the machine reboots NT expects to see the Hard disk in B0 (because the CDROM is on this channel) and fail installation.

## Hanging during install with a DPT Controller

If a large array is created using 4GB drives NT can hanging during installation. To overcome this problem create a small DOS partion on the array then restart the NT installation.

## Windows NT 3.51 hanging during installation on Adaptec based Shogun

A Shogun will hang during the installation of Windows NT 3.51 when loading the SCSI disk driver if more than 2 hard disks are attached to the Adaptec controllers.

To overcome this problem remove all drives apart from the BOOT drive located in B1.

Install Windows NT.  After the installation is complete add the remaining drives and assign /configure them via Disk Administrator.

## Error in Event Log

Event details: THE DRIVER DETECTED A CONTROLLER ERROR ON \DEVICE\SCSIPORT0

This error normally occurs when accessing the CDROM with file manager.

To fix this problem you will need to upgrade the AIC78xx.SYS file.

We recommend to use the file from 7800 Family Manager (Adaptec) Rel 2 or above.

Event details: THE DRIVER DETECTED A CONTROLLER ERROR ON \DEVICE\SCSIPORT1

This error normally occurs when the 2nd SCSI controller is not cabled correctly to the Drive Module.

# NetWare

## Adaptec BIOS Settings

Disable >1GB support

## AIC7870 load order after installation

During the installation of NetWare 4.10 the Primary Adaptec is SLOT 129 and the Secondary Adaptec in Slot 130. However after installation the AIC7870.DSK  need to be loaded in reverse order.

STARTUP.NCF

LOAD AIC7870 SLOT=130

LOAD AIC7870 SLOT=129

## Multiprocessor Shogun with current releases of NetWare

If a Shogun has more than 1 processor installed then the multiprocessor functionality of the machine must be disabled to allow current releases of NetWare to operate correctly.

To disable multiprocessor mode, APICOFF.COM must be run prior to SERVER.EXE.

APICOFF.COM can be obtained via Apricot Insight On-line in File Area 44 or Insight CD release 10.

If the machine has NetWare 4.1 SMP or has been upgraded to run SMP, then APICOFF must be removed.

## NetWare Not Auto Registering Shogun Memory Correctly

NetWare will not automatically register the correct amount of memory in a Shogun unless the machine has been configured with SCU.

If extra memory is added to a Shogun it will automatically address the new memory during POST. However after NetWare has started it will only report the previous amount of memory.

To overcome this run SCU.

Go into the Configuration Settings (Change NO settings).

Save Configuration.

NetWare will then automatically address the correct amount of memory.

# OpenServer 5

## 3Com 3C59x Series Ethernet Cards and SCO OpenServer 5 / UNIX 3.2v4.2

Please be aware that drivers for the 3C59x series of Ethernet cards are now available for both SCO OpenServer 5 and SCO UNIX 3.2v4.2.  Cards in this series include the PCI 3C595 (10/100 Mbit) and the EISA 3C592.

Whilst we have completed a certain amount of testing on OpenServer 5, these drivers are still not officially supported as yet.  We anticipate that SCO will be fully supporting these drivers in the near future by including them in the Advanced Hardware Supplement for Unix 3.2v4.2 and OpenServer 5. Currently therefore, installation and use of these drivers is entirely at the users own risk.

At present, when the drivers are installed, they will be reported by custom as:

AHDK Prototype Network Drivers (ver1.0.0a)

This should be regarded as normal for this release.

When the driver loads at boot time, it should be reported as e3G in the system hardware configuration.

The drivers are available from Insight rel 10 or from Area 39 of Apricot Insight Online and are as follows:

59XSCO3.Z		3c59x Drivers for SCO UNIX (3.2v4.2 and OS3)

59XSCO5.Z		3c59x Drivers for SCO OpenServer 5

## Problem installing SCO OpenServer 5 on Shogun

We have found a problem when installing SCO OpenServer 5 on Shogun systems. On re-booting the system into multi-user mode, it appears to freeze when the graphical login is initialised on tty02 or when the mouse is moved. All of the other console TTY's are then locked. The system then needs to be re-booted.

This is due to the video configuration being set incorrectly during installation. The solution is to boot the system into system maintenance mode, and enter the video configuration manager, by typing:

# scoadmin video

Set the system to use the Cirrus GD542x driver. The problem is typically caused by using the "IBM VGA" option. Select a resolution that the display can handle. If necessary, start off by using a resolution of 640x480. When this is complete, run the system in multi-user mode. The SCO login logo should appear on tty02, and the mouse should also work (ensure that the mouse is selected as high resolution keyboard mouse during initial installation).

If the graphical logo appears distorted or multiple times across the screen, then the resolution and frame rate (Hz) must be changed. The system should not be locked up, so use <ctrl><alt><f1> to get to tty01. Use the video configuration manager as described above to change the parameters.

However, do not alter the video driver from the GD542x setting again, as this will have unpredictable results

Exit from the video configuration manager, and type

# scologin stop

# scologin start

This stops and restarts the graphical login screen on tty02. Use <alt><f2> to return to it. If the display is still not correct, repeat the above until a satisfactory display is achieved.

## Shogun SCO Pack on OpenServer 5

When a full installation of this product is done on a Shogun running OpenServer 5 with a DPT raid configuration, the graphical login on tty02 will be slower to respond. If the software is installed on a Shogun with no network card installed, the graphical screen will almost stop completely, allowing no input on tty02. Other (text) console multiscreens behave normally however, once a changeover has eventually been achieved.

A solution is to only do a partial installation of the SCO pack, and avoid the installation of the raid client. This will be a limitation of the initial releases of the software.

# DPT RAID configuration

## Recommendations

Recommend a maximum of 5 drivers per array of any capacity. Adding more drives degrades performance.

## Single Array Group across 2 Controllers

If you have 2 DPT controllers installed it is not possible to create a Single Array Group spanning both cards.

## !!BEWARE!!

Watch out for moving drives or getting the cabling wrong.  If you boot the system different to the way it was initially installed you will make it appear that you have more than 1 drive failure and will loose your array.

## Mouse Support with DPT Manager

If  MOUSE.EXE version 9.01 is used DPTMGR can possibly lock when loading.

# Tricks

## Flash Disk

<CTRL - D> in flash disk menu returns you to DOS

## SCU

### Locking Configurations

<F8> Locks configurations in SCU

### Increasing speed of Boot

To increase the speed of the machine booting if you have a DPT controller you can disable the on board Adaptec BIOS.

Run SCU

Press <Enter> to select SYSTEM BOARD

Move cursor down to the SCSI ROM BIOS Options Group

Change SCSI-A ROM BIOS scan to “Disable”

Change SCSI-B ROM BIOS scan to “Disable”

Press <Enter> then , <Esc> to return to the Main Menu.

Save Configuration.