Tapes Facts Tricks & Tips

Apricot Shogun using XXPress Motherboard

Tape drives supplied/supported

Conner CTD8000R-S		4-16Gb DDS drive

Conner CTL96G-S		16-64 DDS Autoloader drive

Quantum DLT 4000		20-40Gb DLT drive.

Tape Drive Product codes, part numbers etc.

DRIVE		Product Code	Part No		Media

CTD8000R-S	SU54778	PT15354931	GH51987 (90M Tape pack of 5)

or

XB55638 (120M Tape pack of 5)

CTL96G-S	HA55473	PT15355031	GH51987 (90M Tape pack of 5)

or

XB55638 (120M Tape pack of 5)

DLT4000	HA55474	N/A		XB56600 (20-40Gb Tape pack of 1)

Cleaning Tapes for the 4-16 and 16-64 are available from Apricot under the product

code GH46865.

The DLT drive is supplied by Apricot with a cleaning tape

Firmware Revisions and obtaining updates

The latest versions of the firmware released on the tape drives mentioned in this

document are listed in each section, however from time to time the manufacturers

firmware revisions will supersede Apricot’s.

If a later version of firmware does become available it will be listed in this document.

To obtain firmware updates contact the System Support Group of GTSD.

General tips for using tape drives.

Configuration

Ensure that the Shogun has been setup as per Shogun Facts Tips and Tricks.

Head cleaning and tape selection

In many situations the tape media is the direct cause of read and write errors.

You should retension the tape cartridge regularly to eliminate the possibility of any

alignment problem. If there are any damaged or corrupted data sectors on your

cartridge, you will need to reformat the tape cartridge. You can also try another

cartridge to verify if this is a tape-related problem.

A major cause of backup failure on any tape drive is lack of adhering to a regular

cleaning schedule. All drives should be cleaned after the first use of every new

tape. A regular cleaning schedule will go a long way to prevent problems. Normal

schedule is every 25 to 30 hours of use for 4 and 8mm drives. Refer to your manual

for more specific information about your particular drive.

Use only quality branded media (ensuring that DDS media is used in 4mm systems).

Do NOT use DAT cartridges as they are of a lower quality which will compromise the

integrity of any backups made.

Ensure that software is at latest revisions.

Ensure that the server has had the following upgrades/patches implemented.

Novell Netware

Obtain and implement;

311UP.EXE for Netware 3.11

312UP.EXE for Netware 3.12

410UP.EXE for Netware 4.10

These patch kits are available from area 37 of Insight Online or Insight CD.

Windows NT 3.51

Obtain and implement

SP4_351I.EXE (service pack 4 for NT  3.51)

SP4_351.TXT (Readme file for the above)

This patch kit for NT 3.51 is located in area 88 of Insight Online or Insight CD.

Obtain and implement the latest version of Adaptec 7800 driver located in area 41 of Insight Online or latest version of DPT SCSI driver 7800WNT.EXE located on area 88 of Insight Online/Insight CD.

SCO Openserver 5

Read the in  file area 39 of Insight Online called PATLST.TXT which details current

patches available.

Backup Software

Implement the latest patch kit/updates for you Tape Backup software.

Tape Support and Known Problems

## Tape Support Matrix

| Tape Device | UnixWare | SCO | NT | Netware |
| --- | --- | --- | --- | --- |
| DDS II | 1 | 1 | 1 (2) | 1 (3) |
| DLT 4000 | 1 | 1 | 2 | 3 |
| DDS II autoloader | not supported | 6 | 2 , 4 | 3, 5 |

- Supported in Operating system

- Supported by Arcada Backup Exec ver 6 for NT

- Supported by Arcada Backup Exec ver 7 for Netware

- Supported by Autoloader Arcada Backup Exec supplement for NT

- Supported by Autoloader Arcada Backup Exec supplement for Netware

6.   Supported by SCO’s ARCServe/Open - Apricot product codes SS58355 (Base

Product) with SS58356 (Changer Option)

Autoloaders will work as a “large tape drive” if the supplement is not purchased (not

under OpenServer 5 - ARCServe/Open must be used - see OpenServer 5 section

below).

Netware

Arcserve 5.01g poor backup speeds with DLT

As default Arcserve is configured to use a 512k block size. Due to such a small block

size being used as default the DLT is unable to stream correctly.

To overcome this problem the block size and buffer size need to be increased.

Edit SYS:\ARCSERVE\NLM\TAPESVR.CFG and add the following lines (changes in

Bold).

“SYS:\ARCSERVE\NLM\TAPESVR.CFG”

[CONFIG]

BLOCKSIZE=5

BUFFER=20

[NLM]

NLM1 = standard group-ARCSERVE        ; 1st load

After restarting Arcserve check the block size of the cartridges by the following..

TAPESVR screen - DEVICE MANAGEMENT - TAPE DRIVE STATUS

If the Block size is not 16384 then perform an erase on the tape, this will change the block size from 512 to 16384.

Tape Support and Known Problems Contd

## Arcserve 5.01g and Greater than 16MB ram

To enable Arcserve to use memory above 16Mb the following syntax needs to be

amended to SYS:\SYSTEM\ASTART.NCF (changes in Bold)

SYS:\SYSTEM\ASTART.NCF

REM =======================================================

REM   Make sure the following drivers are loaded before you run ASTART

REM   aha1540.dsk or aha1640.dsk or aha1740.dsk , aspitran.dsk

REM =======================================================

Search Add SYS:\ARCSERVE\NLM

LOAD SYS:\SYSTEM\STREAMS

LOAD SYS:\SYSTEM\CLIB

LOAD SYS:\SYSTEM\TLI

LOAD SYS:\SYSTEM\SPXS

LOAD SYS:\SYSTEM\AFTER311

LOAD SYS:\SYSTEM\NWSNUT

LOAD BTRIEVE -p=4096 -f=20 -h=60 -l=20 -t=15 -u=0 -s=30 -m=512

LOAD BSPXCOM -d=8192 -s=15 -w=3

LOAD SYS:\ARCSERVE\NLM\ASDB

LOAD SYS:\ARCSERVE\NLM\FAXLIB

LOAD SYS:\ARCSERVE\NLM\ARCSERVE

LOAD SYS:\ARCSERVE\NLM\ASPI50 host=0 USEABOVE16

LOAD SYS:\ARCSERVE\NLM\TAPESVR USEABOVE16

LOAD SYS:\ARCSERVE\NLM\CLIENTS

Tape Support and Known Problems Contd

## Arcada Backup EXEC for NT v6.0

The evaluation copy of NT BackupExec v6.0 (LOT NUMBER #DK95108) contains a

DLT driver dated 7/20/95 , size 16480 bytes.

A released copy of BackupExec for NT v 6.0 (LOT NUMBER #DK9590) contains a

DLT driver dated 5/2/95 , size 16096 bytes.

The released copy unfortunately does not work with the DLT 4000 drive on either

Adaptec or DPT controllers. The driver reports that it cannot start in the event log.

The evaluation copy driver works OK. This updated driver DLTTAPE.SYS has been

copied to AREA 44 of Insight Online and the Shogun area of the Insight CD as

DLTNT.EXE.

The DLT driver (DEC / Cipher DLT SCSI drivers , All) is used when adding a tape

device to NT

Also a maintenance pack (BNT6i108.EXE with docs or BNT6L108.EXE no docs) for

Backup EXEC to update to build 108 can be found in AREA 88 of Insight Online

## SCO OpenServer 5

## Can an Autoloader be used as one “large tape drive”?

Under OpenServer 5 the Autoloader is not seen as one “large tape drive”, and will not automatically change tapes when the first one is filled.

The OpenServer 5 tape driver can be enhanced by installing the Advanced Hardware Supplement 5.2 (available form Insight Online area39 or the Insight CD-ROM) to add “jukebox” support.  AHS 5.2 provides a Sjk driver and a “juke” command to manipulate tapes in and out of the tape caddy.  With such a command it would be possible to create a backup script that manipulates tapes in and out of the drive as required.  However, such a script would be complex to create - as for reliability of backups a verification phase is necessary in such a script, and calculating the size of the data to be copied to each tape would be difficult.  Use of this approach is academic at present as the “juke” command fails with the Autoloader supplied by Apricot.  It is hoped that it will be fixed by SCO at some point in the future.

The “Tape Support and Known Problems” section above states that Arcserve/Open from SCO is needed for the Autoloader.  This is the only supported method of using the Autoloader with multiple tapes under OpenServer 5.  The software is easiest to use from it’s graphical interface under X Windows, and is a very comprehensive product indeed.  There are UNIX command line commands, but they are complex, having many command line options and switches.  However, it is fairly easy to “create” a backup command under the graphical interface and “save” it, this can then be used by a UNIX command line command, say, as a cron job for the overnight backup.

# Tape Support and Known Problems Contd

# RAID based Shoguns using cpio and tar to backup data

For RAID based Shogun’s it may be necessary to connect the tape drive to the on-board Adaptec controller by using SCSI cable part number  PT15552431 and 68 pin/50 pin adapter part number PT15552530.  SCSI bus hang problems have been seen on some RAID systems and connecting to the on-board Adaptec for the tape has cured the problem.

## RAID based Shoguns using Arcserve/Open for OpenServer 5

The tape drive MUST be connected to the on-board Adaptec controller as above.  We have seen the Arcserve/Open software cause the DPT card to fail multiple drives within the RAID array if the tape drive is connected to the DPT SCSI bus - this was seen when using an Autoloader, no testing has been done with other tape drives.

## Using DLT 4000 SCSI Tape Drive on SCO OpenServer 5

When doing cpio backups using the DLT 4000 SCSI tape drive under SCO OpenServer 5, it is important to specify a particular block size using the "-C" option. Whilst the device will still back up if this is not done, it will not stream in an efficient manner.

The effect of this is that it's performance will be greatly reduced, with the result that

backups can take up to 7 times longer than is actually necessary.

We therefore recommend that  when cpio is invoked, the blocking factor is set to

10240 bytes to the record. We have tried greater values with no significant

improvement in performance. A typical cpio instruction would be as follows:

# find . -depth -print|cpio -ovC10240 >/dev/rct0

In practice, this has resulted in a data transfer rate approaching 50Mb/min, as against

some 7Mb/min when cpio uses its default blocking factor.

General

In many cases using the default tape backup software supplied with the operating

system may give a good indication as whether it is a hardware or software issue that

you are looking at.

Conner CTD8000R-S	4-16Gb DDS drive

Current Drive Firmware as shipped by Apricot	5.45

Environment			Operating			Non-operating

Temperature			5º-45º C			-40º to 65º C

Relative Humidity		20% to 80% non condensing

Wet Bulb Temperature		78.8º F (26º C)

MTBF	 (20% Duty Cycle)	200,000 hours

MTTR				Less than 30 minutes

Power Requirements

+5V		0.77 A typical

+12V		0.40 A typical

Watts		8.7 typical

Dimensions

Form Factor	5.25" HH

Height		1.7"	43mm

Width		5.8"	147mm

Length		7.1"	180mm

Weight		1.6lbs	0.7Kg

Interpreting the LED indicators

Green		Amber		Drive status

ON		OFF		Cassette inserted

ON		ON		Cassette inserted: tape being read or written

Slow Flashing	ON/OFF	Media warning

Rapid Flashing	ON		Drive could not write to tape correctly

ON/OFF	Rapid Flashing	Hardware error or high humidity

Conner CTL96G-S	16-64Gb DDS Autoloader

Current Drive Firmware as shipped by Apricot	5.75

IPB 2262 does outline the update of the firmware to 5.79 to overcome problems with

Arcserve.

Environment			Operating			Non-operating

Temperature			5º to 45º C			-40º to 65º C

Relative Humidity		20% to 80% non condensing

Wet Bulb Temperature		78.8º F (26º C)

MTBF 				(20% Duty Cycle) 125,000 hours

MTTR				Less than 30 minutes

Power Requirements

+5V	2.2 A typical

+12V	1.7 A typical

Watts	29W for loader typical

Dimensions

Form Factor	5.25" FH

Height	3.2"	82mm

Width	5.7"	146mm

Length	8.0"	203mm

Front Panel

The front panel of the Autoloading DDS-DC drive includes a eight character display, four LEDs and two buttons. The front panel is illustrated below.

Conner 16-64Gb DDS Autoloader Contd

The following table summarizes front-panel LED operation.

LED 			Action 			Meaning

Amber 			ON (lit) 			The drive is reading or writing the tape.

Amber 			Flashing Rapidly 		A hardware fault occurred, or moisture was

detected.

Green(cartridge)		ON (lit) 			A cartridge is inserted & does NOT generate

excess errors.

Green(cartridge)		Flashing Slowly 		A cartridge is inserted but generates

excessive errors beyond a predefined error

threshold. (Warning only) Use a DDS

cleaning cartridge to clean the heads.

Green(cartridge)		Flashing Slowly with	A prerecorded audio cartridge is inserted

amber LED flashing)	and is being played automatically.

Green(cartridge)		Flashing Rapidly 		The drive could not write the tape correctly.

(Error) Use a DDS DAT cleaning cartridge to

clean the heads.

Green(compression)	ON (lit) 			A data compression or decompression operation

is in progress on the current cartridge.

Green (write protect)	ON (lit) 			A write-protected cartridge is currently loaded.

Hardware errors displayed on 8 digit display

Error 10   Magazine Initialization Failure (1st part of Scanning)

Error 11   Magazine Initialization Failure (2nd part of Scanning)

Error 20   Magazine Ejection Failure

Error 30   Magazine Positioning Failure (moving up)

Error 31   Magazine Positioning Failure (moving down)

Error 40   Cassette Insertion Failure

Error 41   Cassette Insertion Failure (can't clamp on)

Error 42   Cassette Insertion Failure (can't clamp off)

Error 50   Cassette Ejection Failure

Error 51   Cassette Ejection Failure  (can't clamp on)

Error 52   Cassette Ejection Failure  (can't clamp off)

Error 60   Drawer Closing Failure

Error 61   Drawer Opening Failure

Error 70   Magazine Position Lost

Error B2   Magazine Initialization Failure (drawer closed)

Error B3   Magazine Initialization Failure (drawer closed)

Error F0   Power-up Failure

The following are "internal" errors and normally should never occur:

Error 03   Command was rejected because magazine is not present.

Error 04   Command was rejected because magazine is not initialized.

Error 05   Command was rejected because invalid slot specified.

Error 06   Command was rejected because cassette is still in drive.

Error 07   Command was rejected because there is no cassette in magazine.

Error 08   Command was rejected because there is no cassette in drive.

Error 09   Command was rejected because specified slot is full.

Conner 16-64Gb DDS Autoloader Contd

Dip switch settings

The settings used by Apricot are as below except that the drives are

set to SCSI id 2.

Conner 16-64Gb DDS Autoloader Contd

Loading cartridges

To load cartridges hold the magazine with the flat side towards you, so that the cartridge slots face away from you. Then insert a cartridge with the write protect tab facing you at the right end of the cartridge.

Inserting a magazine

The drive senses when a four cartridge magazine is used, and automatically closes the drawer and loads the first (top) cartridge. To load a four cartridge magazine:

1	Press the EJECT button to open the drawer. The drawer will open and the

display will indicate that operator action is required.

2	Align the loaded magazine over the open drawer with the flat side of the

magazine towards you, and the protrusions on the sides of the magazine

matching the cut-outs in the drawer.

3	Place the magazine gently in the drawer. Do not force it. The drive senses its

presence, positions it and closes the drawer automatically.

Conner 16-64Gb DDS Autoloader Contd

Head cleaning

There are two points at which the drive heads should be cleaned.

1	As routine maintenance after every 25 hours of read/write activity.

2	Whenever the cartridge status LED flashes.

Note	To clean the heads insert a cleaning cartridge. These are available from

Apricot (GH46865).

Note	The drive automatically recognises the cleaning cartridge, loads it and runs it

for approximately 10 seconds, before it is ejected. Each time the cleaning

cartridge is used a clean unused portion of the tape is advanced. After

approximately 30 uses the entire tape is used and a new cartridge is required.

Quantum DLT 4000		20-40Gb DLT drive.

Current Drive Firmware as shipped by Apricot	CC28

Environment		Operating			Non-operating

Temperature		10°C to 40°C			-40°C to 66°C(ex media)

Relative Humidity	20% to 80%			10% to 95%

Altitude		8,000 ft (2,438 m)			30,000 ft (9,144 m)

Power Requirements

+5V			2.5A Stepping Head Nominal, 3.0A Peak

+12V			0.8A Operational Nominal, 1.5A Peak

Watts			22W Nominal R/W, 33W Operating Max

Dimensions

Form Factor	5.25" FH MD

Height	3.25"	82.5mm

Width	5.7"	145mm

Length	9.6"	243mm

Weight	2.9Kg

Quantum DLT 4000 20-40Gb drive Contd.

The following table summarizes front-panel LED operation

Power  up sequence

1.	Lights on the front panel turn on sequentially top to bottom. All lights stay on for a few  seconds.

Lights on the left of front panel turn on at the same time for about three seconds & then turnoff.

2.	The green operate handle,  the orange Write Protected & yellow Use Cleaning Tape lights turn

off.  The yellow Tape In Use light blinks while the tape drive initializes.

4.	After Initialization, the drive is in one of the following four states:

No cartridge present	1.	The Yellow Tape in Use light turns off.

2.	The green Operate Handle light turns on.

3.	The handle is unlatched.

4.	The drive beeps momentarily.

You can now raise the handle & insert the cartridge.

Cartridge present &		Cartridge loads. When yellow Tape In Use light stops blinking

Handle is down 		& stays on, the tapes actual density is lit. Eg, if tape density is 2.6

then light turns on next to the 2.6 label. When Density Override

blinks, you can select a density.  Drive is ready for use.

Cartridge is present, 	The yellow Tape In Use light turns off.  The green Operate handle

but the handle is up.	light flashes. When you lower the handle, the cartridge loads.

The drive detects an	All left & right side lights blink repeatedly you may try to unload

error condition		tape & reinitialize the drive by pressing the Unload button or turn

power off & on.  Right & left lights stop blinking & the drive tries to

reinitialize, lights turn on steady again & turn off if test succeeds.

Determining the Drive’s Operating Condition.

Light		Colour	State		Operating Condition

Tape In Use	Yellow	Blinking		Tape is moving

On		Tape is loaded ready for use.

Use Cleaning Tape	Yellow	On		Drive needs cleaning or tape is bad

Remains on after	Cleaning tape attempted to clean the drive

unloading	head	but the tape expired, so cleaning was not done

cleaning tape

After cleaning	Problem data cartridge, try another cartridge

turns on again when

you reload cartridge.

Off		Cleaning is complete, or cleaning is not required.

All left/right side lights 	On		POST is starting

Blinking		An error occurred, check termination & cabling of drive

2.6		Yellow	On		Tape is recorded  in 2.6 compression format.

Blinking		Tape is recorded in another density.

6.0		Yellow	On		Tape is recorded in 6.0 format

Blinking		Tape is recorded in another density.

10.0/20.0		Yellow	On		Tape is recorded in 10.0 format

Blinking		Tape is recorded in another density.

Compress	Yellow	On		Compression mode enabled (only in 10.0 density mode)

Off		Compression disabled

Density Override	Yellow	On		You selected a density from the front panel.

Off (default)	Density to be selected by host.

Blinking		You are in density selection mode.

Quantum DLT 4000 20-40Gb drive Contd.

Jumper Settings

SCSI ID Settings

Further Information sources.

Whilst every effort is made to keep this document up to date the following

information sources (from which a fair percentage of this document is taken) are

updated more frequently.

Apricot Insight CD

Apricot Insight On-line 	BBS	0121 717 0444

www.apricot.co.uk/ftp/bbs/g-bbs.htm

Conner/Seagate	www.conner.com

Quantum		www.quantum.com

Adaptec		www.adaptec.com

Cheyenne		www.cheyenne.com